SPECIFICATIONS

SHDC-8700 Series

HDMI 2.0 AOC

DESCRIPTIONS

Our HDMI Active Hybrid Cable is the most competitive long reach HDMI interconnection solution on the market. It supports HDMI Maximum Data Rate 6Gbps with light weight, flexible and ultra-small long-term bending radius (20mm) hybrid cable. With our advanced optical engine embedded. This HDMI Hybrid Cable provides full HDMI signal integrity, support up to 100m maximum length, plug and play. No external power required.

KEY FEATURES

◆ Up to 100m Maximum Length.(Custom Length Available)

◆ Low Power Consumption: 250mW (max). (Power Required)

◆ Plug and Play

◆ Compatible with HDMI 2.0 standard.

◆ Support HDCP2.2,HDR, CEC,ARC

◆ Self-detecting function for EDID information

◆ No loss of signal – delivering crystal clear digital images instantly.

◆ Supports computer resolutions to 1080P and 4K2K(60P).

◆ Hybrid optical cable with fiber and copper wire

Applications

◆ Digital Signage

◆ LED signboards in streets and in stadiums

◆ Medical Imaging Equipment

◆ Airplane On-board Video System

◆ Home Theater

◆ Blue-ray, 3D video, Projector, Set-up box, DVR, Game Consoles and Computer

◆ TV Broadcast Station

◆ Conference Room Video Equipment

◆ Security systems

Absolute Maximum Rating

| Parameter | Symbol | Min | Max | Unit |
| --- | --- | --- | --- | --- |
| Power Supply Voltage | VCC | 4.5 | 5.5 | V |
| Operating Temperature | Top | 0 | 50 | °C |
| Storage Temperature | Ts | -40 | 70 | °C |
| Relative Humidity | Hs | 10 | 80 | %RH |

Recommended Operating Conditions

| Parameter | Symbol | Min | Typical | Max | Unit |
| --- | --- | --- | --- | --- | --- |
| Power Supply Voltage | VCC | 4.75 | 5.0 | 5.25 | V |
| Operating Temperature | Top | 0 |  | 50 | °C |

Specifications

| Item | Description |  |
| --- | --- | --- |
| Units | Transmitter (Source) | Receiver (Sink) |
| HDCP & DDC | HDCP 2.2 Fully Support bi-directional EDID and HDCP communication |  |
| Video Bandwidth | 6 Gbps/x3Channel (max)+ Clock. |  |
| Supported Resolution & Distance | PC: WUXGA 1920 x 1200 up to 100 meters，HDTV : 480p, 720p, 1080i and 1080p deep color, 4K(60P), 3DFull HD 1920 x 1080 up to 100 meters |  |
| O/E Converter | 4 channel 850 nm VCSEL array | 4 CH GaAs PD array |
| Connector Type | HDMI type A |  |
| Cable Type | Hybrid type with OM3 fiber and copper wire |  |
| Cable Length | 15m/20m/30m/40m/50m/60m/70m/80m/90m/100m (with +/-30cm tolerance) Custom length available. |  |
| Cable Outer Diameter | 4.5mm |  |
| Jacket Outer Material | PVC/TPU(Custom) |  |
| Cable Weight TBD | Cable Weight TBD |  |
| Cable Color TBD | Cable Color TBD |  |
| Differential Input | 100 ohm (typ) |  |
| Power Consumption | 250mW (max) |  |
| Power Supply | Non external power required |  |

HDMI Type A Pin  Assignment

HDMI Type A Connector Pin Assignment

| PIN | Function | PIN | Function |
| --- | --- | --- | --- |
| 1 | T.M.D.S. Data 2+ | 11 | TMDS Clock Shield |
| 2 | T.M.D.S. Data 2 Shield | 12 | TMDS Clock- |
| 3 | T.M.D.S. Data 2- | 13 | CEC |
| 4 | T.M.D.S. Data 1+ | 14 | Utility |
| 5 | T.M.D.S. Data 1 Shield | 15 | SCL |
| 6 | T.M.D.S. Data 1- | 16 | SDA |
| 7 | T.M.D.S. Data 0+ | 17 | DDC/CEC Ground |
| 8 | T.M.D.S. Data 2 Shield | 18 | +5V Power |
| 9 | T.M.D.S. Data 0- | 19 | Hot Plug Detect |
| 10 | TMDS Clock+ |  |  |

Installation

To Source, ex. DVD, PC                                        To Sink, ex. TV

Hybrid Cable Profile

4.5

Ordering Information

| Part Number | Description |
| --- | --- |
| SHDC-8700-FM-010 | HDMI AOC，V2.0，4K 2K/60P，Suport EDID/HDCP，Length 10m |
| SHDC-8700-FM-015 | HDMI AOC，V2.0，4K 2K/60P，Suport EDID/HDCP，Length 15m |
| SHDC-8700-FM-020 | HDMI AOC，V2.0，4K 2K/60P，Suport EDID/HDCP，Length 20m |
| SHDC-8700-FM-030 | HDMI AOC，V2.0，4K 2K/60P，Suport EDID/HDCP，Length 30m |
| SHDC-8700-FM-040 | HDMI AOC，V2.0，4K 2K/60P，Suport EDID/HDCP，Length 40m |
| SHDC-8700-FM-050 | HDMI AOC，V2.0，4K 2K/60P，Suport EDID/HDCP，Length 50m |
| SHDC-8700-FM-060 | HDMI AOC，V2.0，4K 2K/60P，Suport EDID/HDCP，Length 60m |
| SHDC-8700-FM-070 | HDMI AOC，V2.0，4K 2K/60P，Suport EDID/HDCP，Length 70m |
| SHDC-8700-FM-080 | HDMI AOC，V2.0，4K 2K/60P，Suport EDID/HDCP，Length 80m |
| SHDC-8700-FM-090 | HDMI AOC，V2.0，4K 2K/60P，Suport EDID/HDCP，Length 90m |
| SHDC-8700-FM-100 | HDMI AOC，V2.0，4K 2K/60P，Suport EDID/HDCP，Length 100m |

Packing Details:

5PCS/CARTON  FOR 70M to 100M

10PCS/CARTON  FOR 10M to 60M

Inner Box                                                Outer Carton

Packing Details Sheet:

| Item No. | Length(m) | Box Size(mm) | CTN Size(cm) | PCS/CTN | N.W. (KG/Box) | N.W. (KG)/CTN | G.W. (KG)/CTN |
| --- | --- | --- | --- | --- | --- | --- | --- |
| SHDC-8700-10 | 10 | 235*225*74 | 47*38.5*25.5 | 10 | 0.7 | 7 | 8 |
| SHDC-8700-15 | 15 | 235*225*74 | 47*38.5*25.5 | 10 | 0.85 | 8.5 | 9.5 |
| SHDC-8700-20 | 20 | 235*225*74 | 47*38.5*25.5 | 10 | 0.95 | 9.5 | 10.5 |
| SHDC-8700-30 | 30 | 235*225*74 | 47*38.5*25.5 | 10 | 1.2 | 12 | 13 |
| SHDC-8700-40 | 40 | 235*225*74 | 47*38.5*25.5 | 10 | 1.45 | 14.5 | 15.5 |
| SHDC-8700-50 | 50 | 285*275*74 | 56.8*40*31 | 10 | 2.05 | 20.5 | 21.5 |
| SHDC-8700-60 | 60 | 285*275*74 | 56.8*40*31 | 10 | 2.25 | 22.5 | 23.5 |
| SHDC-8700-70 | 70 | 335*325*74 | 39*36*34 | 5 | 2.5 | 12.5 | 13 |
| SHDC-8700-80 | 80 | 335*325*74 | 39*36*34 | 5 | 3 | 15 | 16 |
| SHDC-8700-90 | 90 | 335*325*74 | 39*36*34 | 5 | 3.2 | 16 | 17 |
| SHDC-8700-100 | 100 | 335*325*74 | 39*36*34 | 5 | 3.5 | 17.5 | 18.5 |