SPECIFICATIONS

SHDC-8800 Series

HDMI 2.0 detachable D-D AOC

Date:2018-06-19

DESCRIPTIONS

Our new HDMI active Fiber Optic UHD Pluggable HDMI Cable which is suited for critical and versatile installations.It supports HDMI Maximum Data Rate 18Gbps with light weight, flexible and ultra-small long-term bending radius (20mm) hybrid cable. With our advanced optical engine embedded. This HDMI Hybrid Cable provides full HDMI signal integrity, support up to 100m maximum length, plug and play. No external power required.

KEY FEATURES

◆ Supports a wide range of resolutions up to 4K@60Hz (color space 4:4:4) 3D and Deep color

◆ Supports Multi-channel audio, Dolby True HD, DTS-HD Master Audio

◆ Is HDMI compliant: EDID, CEC, HDCP (2.2), HDR (High Dynamic Range)

◆ Reduced EMI and RFI

◆ Has high pulling strength  and compression load

◆ Has Source / Display clearly printed on the connector as well as tagged on the cable for easy identification.

◆ Support 18G super speed

◆ Includes a DC cable to optionally connect an external 5V power supply(if required, this is typically connected on the display side .)

Remark: Cable is hybrid glass fiber and copper , pull strength should be better within 15KG！！！

Applications

SHDC-8800 is ideal for long distance transmission in professional AV systems, indoor and outdoor digital signage and kiosks, home theater systems, surgical theaters ,facility automation systems and whenever high resolution video and audio are required .On the display side, the micro HDMI connector enables smooth pulling of the cable. The SOURCE end connects to the source (for example, a DVD, Blu-ray or game console box) and the DISPLAY end to the acceptor (for example, a projector, an LCD display or a tablet device .

Absolute Maximum Rating

| Parameter | Symbol | Min | Max | Unit |
| --- | --- | --- | --- | --- |
| Power Supply Voltage | VCC | 4.5 | 5.5 | V |
| Operating Temperature | Top | 0 | 50 | °C |
| Storage Temperature | Ts | -40 | 70 | °C |
| Relative Humidity | Hs | 10 | 80 | %RH |

Recommended Operating Conditions

| Parameter | Symbol | Min | Typical | Max | Unit |
| --- | --- | --- | --- | --- | --- |
| Power Supply Voltage | VCC | 4.75 | 5.0 | 5.25 | V |
| Operating Temperature | Top | 0 |  | 50 | °C |

Specifications

| Item | Description |  |
| --- | --- | --- |
| Units | Transmitter (Source) | Receiver (Display) |
| HDCP & DDC | HDCP2.2 Fully Support bi-directional EDID and HDCP communication |  |
| Video Bandwidth | 6 Gbps /x3Channel (max)+ Clock. |  |
| Supported Resolution & Distance | HDTV : 480p, 720p, 1080i and 1080p deep color, 4K@30Hz, 4K@60Hz(4:4:4), 3D Full HD 1920 x 1080 , up to 100 meters |  |
| O/E Converter | 4 channel 850 nm VCSEL array | 4 CH GaAs PD array |
| Connector Type | HDMI type D with HDMI D/Female to HDMI A/Male Adaptor |  |
| Connector Demension | Type D: 44.2mm x 12.7mm x 7.1mm W, D, H Type A : 48.2mm x 20mm x 10mm W, D, H Pulling Tool: 55.2mm x 17.5mm x 13mm W, D, H |  |
| Cable Type | Hybrid type with OM3 fiber and copper wire |  |
| Cable Length | 15m/20m/30m/40m/50m/60m/70m/80m/90m/100m (with +/-30cm tolerance) Custom length available. |  |
| Cable Outer Diameter | 4.5mm |  |
| Jacket Outer Material | PVC/TPU(Custom) |  |
| Cable Weight TBD | Cable Weight TBD |  |
| Cable Color TBD | Cable Color TBD |  |
| Differential Input | 100 ohm (typ) |  |
| Power Consumption | 250mW (max) |  |

HDMI Type A Pin  Assignment

HDMI Type A Connector Pin Assignment

| PIN | Function | PIN | Function |
| --- | --- | --- | --- |
| 1 | T.M.D.S. Data 2+ | 11 | TMDS Clock Shield |
| 2 | T.M.D.S. Data 2 Shield | 12 | TMDS Clock- |
| 3 | T.M.D.S. Data 2- | 13 | CEC |
| 4 | T.M.D.S. Data 1+ | 14 | Utility |
| 5 | T.M.D.S. Data 1 Shield | 15 | SCL |
| 6 | T.M.D.S. Data 1- | 16 | SDA |
| 7 | T.M.D.S. Data 0+ | 17 | DDC/CEC Ground |
| 8 | T.M.D.S. Data 2 Shield | 18 | +5V Power |
| 9 | T.M.D.S. Data 0- | 19 | Hot Plug Detect |
| 10 | TMDS Clock+ |  |  |

Installation

To Source, ex. DVD, PC                                                  To Display, ex. TV

Hybrid Cable Profile

Ordering Information

| Part Number | Description |
| --- | --- |
| SHDC-8800-010 | HDMI AOC，V2.0，4Kx2K/60P，Suport EDID/HDCP，Length 10m |
| SHDC-8800-015 | HDMI AOC，V2.0，4K 2K/60P，Suport EDID/HDCP，Length 15m |
| SHDC-8800-020 | HDMI AOC，V2.0，4K 2K/60P，Suport EDID/HDCP，Length 20m |
| SHDC-8800-030 | HDMI AOC，V2.0，4K 2K/60P，Suport EDID/HDCP，Length 30m |
| SHDC-8800-040 | HDMI AOC，V2.0，4K 2K/60P，Suport EDID/HDCP，Length 40m |
| SHDC-8800-050 | HDMI AOC，V2.0，4K 2K/60P，Suport EDID/HDCP，Length 50m |
| SHDC-8800-060 | HDMI AOC，V2.0，4K 2K/60P，Suport EDID/HDCP，Length 60m |
| SHDC-8800-070 | HDMI AOC，V2.0，4K 2K/60P，Suport EDID/HDCP，Length 70m |
| SHDC-8800-080 | HDMI AOC，V2.0，4K 2K/60P，Suport EDID/HDCP，Length 80m |
| SHDC-8800-090 | HDMI AOC，V2.0，4K 2K/60P，Suport EDID/HDCP，Length 90m |
| SHDC-8800-100 | HDMI AOC，V2.0，4K 2K/60P，Suport EDID/HDCP，Length 100m |

Packing Details:

5PCS/CARTON  FOR 70M to 100M

10PCS/CARTON  FOR 10M to 60M

Inner Box                                                Outer Carton

Packing Details Sheet:

| Item No. | Length(m) | Box Size(mm) | CTN Size(mm) | PCS/CTN | N.W. (KG/Box) | N.W. (KG)/CTN | G.W. (KG)/CTN |
| --- | --- | --- | --- | --- | --- | --- | --- |
| SHDC-8800-10 | 10 | 230*220*74 | 47*38.5*25.5 | 10 | 0.7 | 7 | 8 |
| SHDC-8800-15 | 15 | 230*220*74 | 47*38.5*25.5 | 10 | 0.85 | 8.5 | 9.5 |
| SHDC-8800-20 | 20 | 230*220*74 | 47*38.5*25.5 | 10 | 0.795 | 9.5 | 10.5 |
| SHDC-8800-30 | 30 | 230*220*74 | 47*38.5*25.5 | 10 | 1.2 | 12 | 13 |
| SHDC-8800-40 | 40 | 230*220*74 | 47*38.5*25.5 | 10 | 1.45 | 14.5 | 15.5 |
| SHDC-8800-50 | 50 | 260*250*74 | 56.8*40*31 | 10 | 2.05 | 20.5 | 21.5 |
| SHDC-8800-60 | 60 | 260*250*74 | 56.8*40*31 | 10 | 2.25 | 22.5 | 23.5 |
| SHDC-8800-70 | 70 | 325*315*74 | 39*36*34 | 5 | 2.5 | 12.5 | 13 |
| SHDC-8800-80 | 80 | 325*315*74 | 39*36*34 | 5 | 3 | 15 | 16 |
| SHDC-8800-90 | 90 | 325*315*74 | 39*36*34 | 5 | 3.2 | 16 | 17 |
| SHDC-8800-100 | 100 | 325*315*74 | 39*36*34 | 5 | 3.5 | 17.5 | 18.5 |