SPECIFICATIONS

# SDMP-1400 Series

DP 1.4 Fiber Extender over MPO Optic Cable

General Description

Smartav link  DP1 .4 fiber extender over  MPO optic cable  is a ideal  installation solution for used on home , residential , commercial integration , as well as EMC medical-grade projects . It can be replaced at any time without the need to replace the entire cable . There is no loss of signal over long distance and no EMI interference which make it as the best choice for point-to-point and distributed ultra high definition video connections .

KEY FEATURES

◆ Source / Display DP  1 .4 Fiber Extender

◆ 20cm Long- each side

◆ DP  1 .4 standard

◆ Support 32 .4G Bandwidth with 8K/ 60P , 5K/4K 60P

◆ Support HDCP2 .2

◆ Self-detecting function for EDID information

◆ HDR formats is supported

◆ Up to  100m(330feet) Maximum Length .

◆ Plug and Play

E-like Industrial (HK) LTD.

Tel：+86-755-23224889

Web：www.smartavlink.com

Rev:1.0 1 / 3

MPO Connectors

◆ 12 -Fiber Standard MPO , Polarity Method A

◆ Compatible with MULTIMODE Fiber Optics OM3 OM4 OM5 (cannot work on single mode fiber)

Absolute Maximum Rating

| Parameter | Symbol | Min | Max | Unit |
| --- | --- | --- | --- | --- |
| Power Supply Voltage | VCC | 4 .5 | 5 .5 | V |
| Operating Temperature | Top | 0 | 50 | ° C |
| Storage Temperature | Ts | -40 | 70 | ° C |
| Relative Humidity | Hs | 10 | 80 | % RH |

Recommended Operating Conditions

| Parameter | Symbol | Min | Typical | Max | Unit |
| --- | --- | --- | --- | --- | --- |
| Power Supply Voltage | VCC | 4 .75 | 5 .0 | 5 .25 | V |
| Operating Temperature | Top | 0 |  | 50 | ° C |

Specifications

| Item | Description |  |
| --- | --- | --- |
| Units | Transmitter ( Source) | Receiver ( Sink) |
| HDCP & DDC | HDCP 2 . 2 Fully Support bi-directional EDID and HDCP communication |  |
| Video Bandwidth | 8 . 1 Gbps/x4Channel |  |
| Resolution | 480p , 720p , 1080i and 1080p deep color , 4K50/ 60P , 8K60P |  |
| O/ E Converter | 6 channel 850 nm VCSEL array | 6 CH GaAs PD array |
| Connector Type | DP male and MPO |  |
| Cable Type | OD:3 . 0mm , Multi mode 50/ 125 OM3 - 150 |  |
| Cable Outer Diameter | 3 . 0 mm |  |

E-like Industrial (HK) LTD.

Tel：+86-755-23224889

Web：www.smartavlink.com

Rev:1.0

2 / 3

| Jacket Outer Material | LSZH |
| --- | --- |
| Cable Color | Aqua |
| MPO Insertion loss | ≤ 0 .35dB |
| MPO Return loss | ≥ 30dB |
| Differential Input | 100 ohm |
| Power Consumption | 0 . 3W( Source) 0 .4W( Display) |

Assembly Diagram

Package Included:

1 .   DP1 .4 Fiber Extender (20cm Long- each side )

2 .   MPO Adapter x 2pcs ( MPO fiber cable not included in the package , can order separately if need)

E-like Industrial (HK) LTD.

Tel：+86-755-23224889

Web：www.smartavlink.com

Rev:1.0

3 / 3