SUAM-3000 SERIES

USB 3.0 Active Optical Cable AM-AM

Description

The USB 3.0 Active Optical Cable (AOC) is a USB 3.0 electrical to optical conversion module. This product can translate signals between USB 3.0 electrical and optical signals. AOC is a cabling technology that accepts the same electrical inputs as a traditional copper cable, but uses optical fiber “between the connectors”. AOC uses electrical to optical conversion on the cable ends to improve speed and distance performance of the cable without sacrificing compatibility with standard electrical interface.USB3.0 AOC can extend the transmission distance of USB3.0 transmission to more than 100 meters .AOC offer a quick solution to supply longer cable for USB3.0application.

KEY FEATURES

◆	Extends USB 3.0 super speed devices up to 100 m

◆	Supports all USB 3.0 device types: control, interrupt, bulk and isochronous at up to 10 Gbps

◆	Operates with USB 3.0 super speed host controllers

◆	Supports all major operating systems including Windows, Mac OS, and Linux

◆	Number of devices can be increased using additional hubs

◆	Power adapter at host is not required

◆  Remote unit require standard power to USB3.0 AOC port

◆	Ideal for a wire variety of USB devices, from printer, scanners, hard drives,and audio devices,

to touch screens, web cams, and game controllers

Applications

◆	Industrial Control

◆	Digital Signage

◆	Home Network Integration

◆	Medical Device Connectivity

◆	KVM Extension

◆	Remote Desktop USB 3.0 Connections

◆	Security: web camera, access control

◆	USB 3.0 Device Sharing: print, scan, storage

Applications Reference

SPECIFICATIONS

| Product Name | SUMF-3000 Series |
| --- | --- |
| Description | USB 3.0 Hybrid Active Optical Cable A Male to A Female |
| Connector | USB 3.0 Standard-AMale to A male plug |
| Max. data rate | 10.0Gbps |
| Range | 300 ft( 100 m) over mult-imode optical fiber |
| Optical Transceiver | 850 nm VCSEL (multimode) |
| Cable diameter | 4.0 mm |
| Operating Temperature | 0°C to 50°C (32°F to 122°F) |
| Storage Temperature | -20°C to 70°C (-4°F to 158°F). |
| Humidity | 5% to 90% (non-condensing) |

Absolute Maximum Ratings

| Parameter | Symbol | Minimum | Maximum | Units |
| --- | --- | --- | --- | --- |
| Supply Voltage | VCC | -0.5 | 5.5 | V |
| Operating Temperature | Top | 0 | 70 | °C |
| Storage Temperature | Ts | -40 | 85 | °C |
| Storage Relative Humidity | Hs | 10 | 85 | %RH |

Recommended Operating Conditions

| Parameter | Symbol | Minimum | Typical | Maximum | Units |
| --- | --- | --- | --- | --- | --- |
| Supply Voltage | VCC | 4.75 | 5 | 5.25 | V |
| Operating Temperature | Top | 0 |  | 50 | °C |

Characteristics

| Parameter | Description | Minimum | Typical | Maximum | Units |
| --- | --- | --- | --- | --- | --- |
| VCC | Supply Voltage | 4.75 | 5 | 5.25 | V |
| IBUS | Current at USB Bus Power | 49.5 | 55 | 60.5 | mA |
| RDIFF | Differential impedance |  | 90 |  | Ω |
| USB3.0 AOC TX Characteristic |  |  |  |  |  |
| PTX | Optical TX output power | 0 |  |  | dBm |
| Rj | Random jitter |  |  | 3 | ps |
| Dj | Deterministic jitter |  |  | 60 | ps |
| Tj | Total jitter at a 10-12 BER |  |  | 100 | ps |
| USB3.0 AOC RX Characteristic |  |  |  |  |  |
| SRX | RX sensitivity | -7.5 |  |  | dBm |
| CAC-COUPLING | AC coupling capacitor on RXOUT | 75 | 100 | 200 | nF |

USB 3.0 Standard-A Plug Pin Assignment

USB 3.0 Standard-A Plug Pin Assignment

| Pin number | Signal Name | Description |
| --- | --- | --- |
| 1 | VBUS | Power |
| 2 | D- | USB 2.0 differential pair |
| 3 | D+ |  |
| 4 | GND | Ground for power return |
| 5 | SSTX- | SuperSpeed transmitter differential pair |
| 6 | SSTX+ |  |
| 7 | GND | Ground for signal return |
| 8 | SSRX- | SuperSpeed receiver differential pair |
| 9 | SSRX+ |  |
| Shell | Shield | Shield |

Drawing

Ordering Information

| Part Number | Description |
| --- | --- |
| SUAM-3000-010 | USB 3.0 AOC AM-AM, 5Gbps,Length 10m |
| SUAM-3000-015 | USB 3.0 AOC AM-AM, 5Gbps,Length 15m |
| SUAM-3000-020 | USB 3.0 AOC AM-AM, 5Gbps,Length 20m |
| SUAM-3000-030 | USB 3.0 AOC AM-AM, 5Gbps,Length 30m |
| SUAM-3000-050 | USB 3.0 AOC AM-AM, 5Gbps,Length 50m |
| SUAM-3000-070 | USB 3.0 AOC AM-AM, 5Gbps,Length 70m |
| SUAM-3000-080 | USB 3.0 AOC AM-AM, 5Gbps,Length 80m |
| SUAM-3000-100 | USB 3.0 AOC AM-AM, 5Gbps,Length 100m |