SUAM-3120 Series New Solution

USB 3.0 (Compatible with USB 2.0) AOC

Description

The USB 3.0 Active Optical Cable (AOC) is a USB 3.0 electrical to optical conversion module. This product can translate signals between USB 3.0 electrical and optical signals. AOC is a cabling technology that accepts the same electrical inputs as a traditional copper cable, but using optical fiber“between the connectors”. The USB fiber cable can be up to 15m with USB 3.0 version and also downward compatible with USB 2.0 and USB 1.1 at Device Side.

KEY FEATURES

◆	Extends USB 3.0 & USB 2.0 super speed devices up to 15 m

◆	Supports all USB 3.0 device types at Host: control, interrupt, bulk and isochronous at up to 10 Gbps

◆	Operates with USB 3.0 super speed host controllers

◆	Supports Windows & Mac operating systems

◆	Number of devices can be increased using additional hubs

◆	A substantial increase in USB2.0 device bandwidth with special technology

◆	Ideal for a wire variety of USB devices, from printer, scanners, hard drives,and audio devices,

to touch screens, web cams, and game controllers

Applications

◆	Industrial Control

◆	Digital Signage

◆	Home Network Integration

◆	Medical Device Connectivity

◆	KVM Extension

◆	Remote Desktop USB 3.0 and USB2.0 Connections

◆	Security: web camera, access control

◆	USB 3.0 & USB2.0 Device Sharing: print, scan, storage

SPECIFICATIONS

| Product Name | SUAM-3120 Series |
| --- | --- |
| Description | USB 3.0 Active Optical Cable downward compatible with USB 2.0 |
| Connector | USB 3.0 Standard-A Male to A male plug |
| Max. data rate | 10.0Gbps |
| Range | 50 ft( 15 m) over multimode optical fiber |
| Optical Transceiver | 850 nm VCSEL (multimode) |
| Cable diameter | 4.8 mm |
| Operating Temperature | 0°C to 50°C (32°F to 122°F) |
| Storage Temperature | -20°C to 70°C (-4°F to 158°F). |
| Humidity | 5% to 90% (non-condensing) |

Absolute Maximum Ratings

| Parameter | Symbol | Minimum | Maximum | Units |
| --- | --- | --- | --- | --- |
| Supply Voltage | VCC | -0.5 | 5.5 | V |
| Operating Temperature | Top | 0 | 50 | °C |
| Storage Temperature | Ts | -20 | 70 | °C |
| Storage Relative Humidity | Hs | 10 | 85 | %RH |

Recommended Operating Conditions

| Parameter | Symbol | Minimum | Typical | Maximum | Units |
| --- | --- | --- | --- | --- | --- |
| Supply Voltage | VCC | 4.75 | 5 | 5.25 | V |
| Operating Temperature | Top | 0 |  | 50 | °C |

Characteristics

| Parameter | Description | Minimum | Typical | Maximum | Units |
| --- | --- | --- | --- | --- | --- |
| VCC | Supply Voltage | 4.75 | 5 | 5.25 | V |
| IBUS | Current at USB Bus Power | 49.5 | 55 | 60.5 | mA |
| RDIFF | Differential impedance |  | 90 |  | Ω |
| USB3.0 AOC TX Characteristic |  |  |  |  |  |
| PTX | Optical TX output power | 0 |  |  | dBm |
| Rj | Random jitter |  |  | 3 | ps |
| Dj | Deterministic jitter |  |  | 60 | ps |
| Tj | Total jitter at a 10-12 BER |  |  | 100 | ps |
| USB3.0 AOC RX Characteristic |  |  |  |  |  |
| SRX | RX sensitivity | -7.5 |  |  | dBm |
| CAC-COUPLING | AC coupling capacitor on RXOUT | 75 | 100 | 200 | nF |

Ordering Information

| Part Number | Description |
| --- | --- |
| SUAM-3120-003 | USB 3.0&2.0 AOC, 10Gbps,Length 3m |
| SUAM-3120-005 | USB 3.0&2.0 AOC, 10Gbps,Length 5m |
| SUAM-3120-008 | USB 3.0&2.0 AOC, 10Gbps,Length 8m |
| SUAM-3120-010 | USB 3.0&2.0 AOC, 10Gbps,Length 10m |
| SUAM-3120-015 | USB 3.0&2.0 AOC, 10Gbps,Length 15m |