SPECIFICATIONS

SHDC-9810 Series

Super Slim HDMI 2.1 AOC

DESCRIPTIONS

Our super slim HDMI Active Hybrid Cable is the most competitive long reach HDMI interconnection solution on the market ,only OD3.4mm. It supports HDMI Maximum Data Rate 12Gbps per channel and total 48Gbps bandwidth with light weight, flexible and ultra-small long-term bending radius (20mm) hybrid cable. With our advanced optical engine embedded. This HDMI Hybrid Cable provides full HDMI signal integrity, support up to 10m maximum length, plug and play. No external power required.

KEY FEATURES

◆ Super slim ,OD3.4mm

◆ Up to 10m Maximum Length.(Custom Length Available)

◆ Plug and Play

◆ Compatible with HDMI 2.1 standard.

◆ Backward compatible with HDMI2.0/HDMI1.4

◆ Support HDCP2.2

◆ Support (HDR) Dynamic HDR formats is supported.

◆ Support (VRR) Variable refresh rate

◆ Support (QMS) Quick media switching

◆ Support (QFT) Quick frame transport

◆ Support (ALLM) Auto low latency Mode

◆ Support (DSC) Display Stream Compression

◆ Support 48Gbps Bandwidth with 4Kp50/60/100/120 and 8Kp50/60

◆ Self-detecting function for EDID information

◆ No loss of signal – delivering crystal clear digital images instantly.

◆ Hybrid optical cable with fiber and copper wire

Applications

◆ Digital Signage

◆ LED signboards in streets and in stadiums

◆ Medical Imaging Equipment

◆ Airplane On-board Video System

◆ Home Theater

◆ Blue-ray, 3D video, Projector, Set-up box, DVR, Game Consoles and Computer

◆ TV Broadcast Station

◆ Conference Room Video Equipment

◆ Security systems

Absolute Maximum Rating

| Parameter | Symbol | Min | Max | Unit |
| --- | --- | --- | --- | --- |
| Power Supply Voltage | VCC | 4.5 | 5.5 | V |
| Operating Temperature | Top | 0 | 50 | °C |
| Storage Temperature | Ts | -40 | 70 | °C |
| Relative Humidity | Hs | 10 | 80 | %RH |

Recommended Operating Conditions

| Parameter | Symbol | Min | Typical | Max | Unit |
| --- | --- | --- | --- | --- | --- |
| Power Supply Voltage | VCC | 4.75 | 5.0 | 5.25 | V |
| Operating Temperature | Top | 0 |  | 50 | °C |

Specifications

| Item | Description |  |
| --- | --- | --- |
| Units | Transmitter (Source) | Receiver (Sink) |
| HDCP & DDC | HDCP 2.2 Fully Support bi-directional EDID and HDCP communication |  |
| Video Bandwidth | 12 Gbps/x4Channel |  |
| Supported Resolution & Distance | 480p, 720p, 1080i and 1080p deep color, 4Kp50/60/100/120 and 8Kp50/60 up to 10m |  |
| O/E Converter | 4 channel 850 nm VCSEL array | 4 CH GaAs PD array |
| Connector Type | HDMI type A |  |
| Cable Type | Hybrid type with OM3 fiber and copper wire |  |
| Cable Length | 1m/2m/3m/5m/7.5m/10m (with +/-20cm tolerance) Custom length available. |  |
| Cable Outer Diameter | 3.4mm |  |
| Jacket Outer Material | PVC/TPU(Custom) |  |
| Cable Weight TBD | Cable Weight TBD |  |
| Cable Color TBD | Cable Color TBD |  |
| Differential Input | 100 ohm (typ) |  |
| Power Consumption | 250mW (max) |  |
| Power Supply | Non external power required |  |

Installation

To Source, ex. DVD, PC                                        To Sink, ex. TV

Hybrid Cable Profile

4.5

Ordering Information

| Part Number | Description |
| --- | --- |
| SHDC-9810--001 | HDMI AOC，V2.1，8K 4K/60P，Suport EDID/HDCP，Length 1m |
| SHDC-9810--002 | HDMI AOC，V2.1，8K 4K/60P，Suport EDID/HDCP，Length 2m |
| SHDC-9810--003 | HDMI AOC，V2.1，8K 4K/60P，Suport EDID/HDCP，Length 3m |
| SHDC-9810--005 | HDMI AOC，V2.1，8K 4K/60P，Suport EDID/HDCP，Length 5m |
| SHDC-9810--010 | HDMI AOC，V2.1，8K 4K/60P，Suport EDID/HDCP，Length 10m |