SPECIFICATIONS

SHMP-8700 Series

MPO Pure Fiber Cable with HDMI 4K AOC

Description

Smartavlink detachable MPO pure fiber HDMI cable is with our advanced optical engine embedded technology. It is using optical fiber between the standard MPO connectors. No EMI and RFI. It supports HDMI 18Gbps bandwidth with light weight, flexible to long distance, specialized for cases which need to pass through the tube. Minimum Bending radius is 30mm. Detachable design is more conducive to later maintenance and product upgrading.

Key Features

◆ MPO port

◆ Up to 100m Maximum Length.(Custom Length Available)

◆ Plug and Play

◆ HDMI2.0 standard

◆ Support HDCP2.3

◆ Support 18G Bandwidth with4K/50P/60P

◆ Self-detecting function for EDID information

◆ HDR formats is supported

Applications

◆ Digital Signage

◆ LED signboards in streets and in stadiums

◆ Medical Imaging Equipment

◆ Airplane On-board Video System

◆ Home Theater

◆ Blue-ray, 3D video, Projector, Set-up box, DVR, Game Consoles and Computer

◆ TV Broadcast Station

◆ Conference Room Video Equipment

◆ Security systems

Absolute Maximum Rating

| Parameter | Symbol | Min | Max | Unit |
| --- | --- | --- | --- | --- |
| Power Supply Voltage | VCC | 4.5 | 5.5 | V |
| Operating Temperature | Top | 0 | 50 | °C |
| Storage Temperature | Ts | -40 | 70 | °C |
| Relative Humidity | Hs | 10 | 80 | %RH |

Recommended Operating Conditions

| Parameter | Symbol | Min | Typical | Max | Unit |
| --- | --- | --- | --- | --- | --- |
| Power Supply Voltage | VCC | 4.75 | 5.0 | 5.25 | V |
| Operating Temperature | Top | 0 |  | 50 | °C |

Specifications

| Item | Description |  |
| --- | --- | --- |
| Units | Transmitter (Source) | Receiver (Sink) |
| HDCP & DDC | HDCP 2.2 Fully Support bi-directional EDID and HDCP communication |  |
| Video Bandwidth | 6 Gbps x 3Channel |  |
| Supported Resolution & Distance | 480p, 720p, 1080i and 1080p deep color, 4Kp50/60 up to 100m |  |
| O/E Converter | 6 channel 850 nm VCSEL array | 6 CH GaAs PD array |
| Connector Type | HDMI type A and MPO |  |
| Cable Type | OD:3.0mm, Multimode 50/125 OM3-150, 8Fibers Micro Cable |  |
| Cable Length | 5m/10m/15m/20m/25m/30m/40m/50m/60m/70m/80m/90m/100m (with +/-30cm tolerance) Custom length available. |  |
| Cable Outer Diameter | 3.0mm |  |
| Jacket Outer Material | LSZH(Custom) |  |
| Cable Color | Aqua、Black |  |
| MPO Insertion loss | ≤0.35dB |  |
| MPO Return loss | ≥30dB |  |
| Differential Input | 100 ohm |  |
| Power Consumption | 0.3W(Source) / 0.4W(Display) |  |

Assembly Diagram

4.5

MPO Pure fiber cable (Length Customized)

KeyUp

KeyUp

MPO male connector

MPO male connector

HDMI2.0 AM connector (Source)

HDMI2.0 AM connector (Display)

Adapter

Adapter

USB Power supply cable

Structure Chart