# SPECIFICATIONS

SHPC-8700 Series

HDMI 2.0 Pure Fiber Cable

Descriptions

Smart av link  Pure  Fiber  HDMI Cable  is the  most competitive  long  reach  HDMI  interconnection solution on  the  market.  It  supports  HDMI  Maximum  Data  Rate  6Gbps  with  light  weight ,  flexible  and  ultra- small long-term  bending  radius  (20mm)  hybrid  cable .  With  our  advanced  optical  engine  embedded.  OD3 . 0mm super slim , for cases request no EMI , supporting up to  100m maximum length , plug and play .

E-like Industrial (HK) LTD.

Tel：+86-755-23200789

Web：www.smartavlink.com

Rev:1.0

1 / 4

Key Features

◆ Up to  100m Maximum Length ( Custom Length Available)

◆ Request RX side ( Display Device) self-powered

◆ Plug and Play

◆ Compatible with HDMI 2 . 0 standard

◆ Support HDCP2 .2 , HDR , CEC ,ARC

◆ Self-detecting function for EDID information

◆ No loss of signal – delivering clear digital images instantly

◆ Support computer resolutions to  1080P and 4K2K (60P)

Applications

◆ Digital Signage

◆ LED signboards in streets and in stadiums

◆ Medical Imaging Equipment

◆ Airplane On -board Video System

◆ Home Theater

◆ Blue - ray , 3D video , Projector , Set- up box , DVR , Game Consoles and Computer

◆ TV Broadcast Station

◆ Conference Room Video Equipment

◆ Security systems

Absolute Maximum Rating

| Parameter | Symbol | Min | Max | Unit |
| --- | --- | --- | --- | --- |
| Power Supply Voltage | VCC | 4 .5 | 5.5 | V |
| Operating Temperature | Top | 0 | 50 | ° C |
| Storage Temperature | Ts | -40 | 70 | ° C |
| Relative Humidity | Hs | 10 | 80 | % RH |

Recommended Operating Conditions

| Parameter | Symbol | Min | Typical | Max | Unit |
| --- | --- | --- | --- | --- | --- |
| Power Supply Voltage | VCC | 4 .75 | 5 .0 | 5 .25 | V |
| Operating Temperature | Top | 0 |  | 50 | ° C |

E-like Industrial (HK) LTD.

Tel：+86-755-23200789

Web：www.smartavlink.com

Rev:1.0 2 / 4

Specifications

| Item | Description |  |
| --- | --- | --- |
| Units | Transmitter ( Source) | Receiver ( Sink) |
| HDCP & DDC | HDCP 2 . 2 Fully Support bi-directional EDID and HDCP communication |  |
| Video Bandwidth | 6 Gbps/x3Channel ( max) + Clock . |  |
| Supported Resolution & Distance | PC: WUXGA 1920 x 1200 up to 100 meters ，HDTV : 480p , 720p , 1080i and 1080p deep color , 4K( 60P) , 3DFull HD 1920 x 1080 up to 100 meters |  |
| O/ E Converter | 4 channel 850nm VCSEL array | 4 CH GaAs PD array |
| Connector Type | HDMI type A |  |
| Cable Type | OM3 fiber |  |
| Cable Length | 15m/ 20m/ 30m/40m/ 50m/ 60m/ 70m/ 80m/ 90m/ 100m (with +/ - 30cm tolerance) Custom length available . |  |
| Cable Outer Diameter | 3 . 0mm |  |
| Jacket Outer Material | LSZH |  |
| Cable Weight TBD | Cable Weight TBD |  |
| Cable Color TBD | Cable Color TBD |  |
| Differential Input | 100 ohm (typ) |  |
| Power Consumption | 250mW ( max) |  |
| Power Supply | Request Display Device self-powered |  |

HDMI Type A Pin Assignment

HDMI Type A Connector Pin Assignment

E-like Industrial (HK) LTD.

Tel：+86-755-23200789

Web：www.smartavlink.com

Rev:1.0

3 / 4

| PIN | Function | PIN | Function |
| --- | --- | --- | --- |
| 1 | T . M . D .S . Data 2+ | 11 | TMDS Clock Shield |
| 2 | T . M . D .S . Data 2 Shield | 12 | TMDS Clock- |
| 3 | T . M . D .S . Data 2- | 13 | CEC |
| 4 | T . M . D .S . Data 1+ | 14 | Utility |
| 5 | T . M . D .S . Data 1 Shield | 15 | SCL |
| 6 | T . M . D .S . Data 1- | 16 | SDA |
| 7 | T . M . D .S . Data 0+ | 17 | DDC/ CEC Ground |
| 8 | T . M . D .S . Data 2 Shield | 18 | +5V Power |
| 9 | T . M . D .S . Data 0- | 19 | Hot Plug Detect |
| 10 | TMDS Clock+ |  |  |

# Ordering Information

| Part Number | Description |
| --- | --- |
| SHPC- 8700- 010 | HDMI AOC ，V2 .0 ，4K 2K/ 60P ，Support EDID/ HDCP ，Length 10m |
| SHPC- 8700- 015 | HDMI AOC ，V2 .0 ，4K 2K/ 60P ，Support EDID/ HDCP ，Length 15m |
| SHPC- 8700- 020 | HDMI AOC ，V2 .0 ，4K 2K/ 60P ，Support EDID/ HDCP ，Length 20m |
| SHPC- 8700- 030 | HDMI AOC ，V2 .0 ，4K 2K/ 60P ，Support EDID/ HDCP ，Length 30m |
| SHPC- 8700- 040 | HDMI AOC ，V2 .0 ，4K 2K/ 60P ，Support EDID/ HDCP ，Length 40m |
| SHPC- 8700- 050 | HDMI AOC ，V2 .0 ，4K 2K/ 60P ，Support EDID/ HDCP ，Length 50m |
| SHPC- 8700- 060 | HDMI AOC ，V2 .0 ，4K 2K/ 60P ，Support EDID/ HDCP ，Length 60m |
| SHPC- 8700- 070 | HDMI AOC ，V2 .0 ，4K 2K/ 60P ，Support EDID/ HDCP ，Length 70m |
| SHPC- 8700- 080 | HDMI AOC ，V2 .0 ，4K 2K/ 60P ，Support EDID/ HDCP ，Length 80m |
| SHPC- 8700- 100 | HDMI AOC ，V2 . 0 ，4K 2K/ 60P ，Support EDID/ HDCP ，Length 100m |

E-like Industrial (HK) LTD.

Tel：+86-755-23200789

Web：www.smartavlink.com

Rev:1.0 4 / 4