# SPECIFICATIONS

SSHC-9810 Series

HDMI 2.1 Super Slim AOC

DESCRIPTIONS

Sav link HDMI 2 . 1 super slim active hybrid cable is the most thin , flexible HDMI interconnection solution on the  market , cable  is extremely thin  and only  OD4 . 0mm .  It supports  HDMI  Maximum  Data  Rate  12Gbps per channel and total 48Gbps bandwidth with light weight and ultra -small long-term bending radius (20mm) hybrid  cable .  With  our  advanced  optical  engine  embedded.  This  HDMI  Hybrid  Cable  provides  full  HDMI

signal integrity , support up to  10m maximum length , plug and play .

KEY FEATURES

◆ Hybrid optical cable with fiber and copper wire

◆ Super slim , OD3 .4mm

◆ Compatible with HDMI 2 . 1 standard , backward compatible with HDMI2 .0/ HDMI1 .4

◆ Support 48Gbps Bandwidth with 4Kp50/ 60/ 100/ 120 and 8Kp50/ 60

◆ Support HDCP2 .2

◆ Support ( HDR) Dynamic HDR formats

◆ Support (VRR) Variable refresh rate

E-like Industrial (HK) LTD.

Tel：+86-755-23224889

Web：www.smartavlink.com

Rev:1.0 1 / 4

◆ Support (QMS) Quick media switching

◆ Support (QFT) Quick frame transport

◆ Support (ALLM) Auto low latency Mode

◆ Support (DSC) Display Stream Compression

◆ Self-detecting function for EDID information

◆ Support eARC

◆ No loss of signal – delivering crystal clear digital images instantly

◆ Up to  10m Maximum Length

◆ Plug and Play , no external power required

Applications

◆ Digital Signage

◆ LED signboards in streets and in stadiums

◆ Medical Imaging Equipment

◆ Airplane On -board Video System

◆ Home Theater

◆ Blue - ray , 3D video , Projector , Set- up box , DVR , Game Consoles and Computer

◆ TV Broadcast Station

◆ Conference Room Video Equipment

◆ Security systems

Absolute Maximum Rating

| Parameter | Symbol | Min | Max | Unit |
| --- | --- | --- | --- | --- |
| Power Supply Voltage | VCC | 4 .5 | 5 .5 | V |
| Operating Temperature | Top | 0 | 50 | ° C |
| Storage Temperature | Ts | -40 | 70 | ° C |
| Relative Humidity | Hs | 10 | 80 | % RH |

E-like Industrial (HK) LTD.

Tel：+86-755-23224889

Web：www.smartavlink.com

Rev:1.0 2 / 4

Recommended Operating Conditions

| Parameter | Symbol | Min | Typical | Max | Unit |
| --- | --- | --- | --- | --- | --- |
| Power Supply Voltage | VCC | 4 .75 | 5 .0 | 5 .25 | V |
| Operating Temperature | Top | 0 |  | 50 | ° C |

Specifications

| Item | Description |  |
| --- | --- | --- |
| Units | Transmitter ( Source) | Receiver ( Sink) |
| HDCP & DDC | HDCP 2 . 2 Fully Support bi-directional EDID and HDCP communication |  |
| Video Bandwidth | 12 Gbps/x4Channel |  |
| Supported Resolution & Distance | 480p , 720p , 1080i and 1080p deep color , 4Kp50/ 60/ 100/ 120 and 8Kp50/ 60 up to 10m |  |
| O/ E Converter | 4 channel 850 nm VCSEL array | 4 CH GaAs PD array |
| Connector Type | HDMI type A |  |
| Cable Type | Hybrid type with OM3 fiber and copper wire |  |
| Cable Length | 1m/2m/ 3m/ 5m/ 7m/ 10m (with +/ - 20cm tolerance) Custom length available . |  |
| Cable Outer Diameter | 4 . 0 mm |  |
| Jacket Outer Material | PVC |  |
| Cable Color | Black |  |
| Differential Input | 100 ohm (typ) |  |
| Power Consumption | 250mW ( max) |  |
| Power Supply | Non external power required |  |

E-like Industrial (HK) LTD.

Tel：+86-755-23224889

Web：www.smartavlink.com

Rev:1.0

3 / 4

Installation

To Source , ex . DVD , PC                                                                                               To  Sink ,  ex .  TV

Hybrid Cable Profile

# Ordering Information

| Part Number | Description |
| --- | --- |
| SSHC- 9810- 001 | HDMI AOC， V2 . 1， 8K 4K/ 60P， Support EDID/ HDCP， Length 1m |
| SSHC- 9810- 002 | HDMI AOC， V2 . 1， 8K 4K/ 60P， Support EDID/ HDCP， Length 2m |
| SSHC- 9810- 003 | HDMI AOC， V2 . 1， 8K 4K/ 60P， Support EDID/ HDCP， Length 3m |
| SSHC- 9810- 005 | HDMI AOC， V2 . 1， 8K 4K/ 60P， Support EDID/ HDCP， Length 5m |
| SSHC- 9810- 007 | HDMI AOC， V2 . 1， 8K 4K/ 60P， Support EDID/ HDCP， Length 7m |
| SSHC- 9810- 010 | HDMI AOC， V2 . 1， 8K 4K/ 60P， Support EDID/ HDCP， Length 10m |

E-like Industrial (HK) LTD.

Tel：+86-755-23224889

Web：www.smartavlink.com

Rev:1.0 4 / 4